Gemini: A Temporally Constrained Variance-Based Pipeline for Automated Scratch-Wound Healing Quantification

[Author list]

Department of [Department], Icahn School of Medicine at Mount Sinai, New York, NY

Correspondence: [Corresponding author email]

# Abstract

Scratch-wound healing assays are a cornerstone of in vitro cell migration studies, yet quantification remains largely manual and prone to inter-observer variability. We present Gemini, a fully automated computational pipeline that segments wound boundaries from phase-contrast time-lapse microscopy and enforces a biologically motivated monotonic closure constraint across the temporal sequence.

The pipeline operates in two stages: (1) a single-frame detector that extracts wound boundaries via local variance analysis, RANSAC-based outlier rejection, and Savitzky–Golay smoothing, followed by (2) a stateful temporal integrator that constrains per-column edge positions to move only inward over time, encoding the axiom that scratch wounds can only heal. The system requires no machine-learning training data, operates on standard brightfield or phase-contrast images, and processes a typical 20-frame trajectory in under 10 seconds on commodity hardware. We describe the mathematical formulation, algorithmic design, quality-control criteria, and downstream metric extraction. A multi-tier validation comprising geometric plausibility checks, distributional analysis of baseline masks, and optional manual curation ensures robust performance across diverse experimental conditions. The pipeline is implemented in Python 3.11 with fully vectorized NumPy operations and supports parallel processing of independent biological samples. We provide the complete open-source implementation together with configuration defaults tuned for 10×-magnification scratch-wound images of approximately 1024 × 1280 pixels.

# 1. Introduction

## 1.1 The Scratch-Wound Assay

The scratch-wound (or wound-healing) assay is one of the most widely used in vitro methods for studying collective cell migration (Liang et al., 2007; Jonkman et al., 2014). A confluent cell monolayer is mechanically disrupted—typically by drawing a pipette tip across the surface—creating a cell-free gap. Time-lapse phase-contrast microscopy then records the progressive closure of this gap as cells at the wound margin proliferate and migrate inward. The rate and completeness of wound closure serve as proxies for migratory capacity and are routinely used to evaluate drug effects, gene knockdowns, and phenotypic differences between cell lines.

## 1.2 Limitations of Current Approaches

Despite its conceptual simplicity, quantifying wound healing remains a bottleneck. Manual measurement of wound area from exported images introduces inter-observer variability, is labor-intensive (especially for multi-well, multi-timepoint experiments), and does not scale to the hundreds of trajectories generated by modern high-content screening platforms. Several computational tools have been proposed—including TScratch (Gebäck et al., 2009), ImageJ macros (Suarez-Arnedo et al., 2020), and deep-learning segmenters (Vu et al., 2023)—but they share common limitations:

Single-frame independence. Most tools process each image in isolation, ignoring the temporal structure of the experiment and producing noisy area-versus-time curves with non-biological fluctuations (apparent wound re-opening).

Training data requirements. Machine-learning approaches require annotated ground truth masks, which themselves suffer from observer bias.

Lack of biological constraints. No existing tool enforces the physical reality that a properly conducted scratch wound can only close over time.

## 1.3 Our Contribution

We introduce Gemini, a two-stage pipeline that addresses these limitations:

(1) A variance-based single-frame detector that exploits the fundamental texture difference between cell monolayer (high local variance due to cell edges, organelles, and phase-contrast halos) and the wound channel (smooth, low variance). No training data or threshold tuning by the user is required.

(2) A stateful temporal integrator that encodes the monotonic closure axiom directly into the segmentation: once a pixel column has been declared healed, it cannot revert. This eliminates non-biological noise in the area-versus-time curve and produces smooth, strictly non-increasing wound trajectories.

The pipeline is complemented by automated quality control, parallel trajectory processing, and downstream metric extraction (closure speed, edge displacement, closing fraction), providing a complete end-to-end solution from raw microscopy images to publication-ready quantitative data.

# 2. Methods

## 2.1 Problem Formulation

### Notation

Let I(x, y, t) ∈ [0, 1] denote the intensity-normalized grayscale image at spatial position (x, y) and discrete timepoint t ∈ {0, 1, …, T}. The image has dimensions H × W (rows × columns). We seek a binary wound mask M(x, y, t) = 1 if pixel (x, y) is wound at time t, and 0 otherwise; and per-column edge functions:

ux, t=miny : Mx, y, t= 1       (upper edge)

l(x, t) = max{y : M(x, y, t) = 1}    (lower edge)

where y increases downward in image coordinates.

### Wound Area

The total wound area at time t is the sum over all columns of the wound width at each column: At= Σmax0, lx,t– ux,t+ 1  A(t) = Σ max(0, ℓ(x,t) − u(x,t) + 1) for x = 0, …, W−1.

### Biological Axiom (Monotonic Closure)

In a properly conducted scratch-wound assay, wound area is a monotonically non-increasing function of time:

A(t+1) ≤ A(t)    for all t ≥ 0

We enforce this constraint at the per-column edge level (Section 2.3), which is strictly stronger than the aggregate area constraint.

## 2.2 Single-Frame Detection

The single-frame detector (WoundDetector) implements a seven-stage pipeline that transforms a raw grayscale image into a validated wound mask with smooth boundary curves. Each stage is described below.

Stage 1: Local Variance Texture Map. Cell monolayers exhibit high spatial variance due to cell borders, nuclei, and phase-contrast halos. The wound channel, lacking cells, is comparatively smooth. We use local variance as a texture discriminator. For a square window of side length w (default w = 15), the local variance at each pixel is computed via the uniform-filter identity: σ²(x, y) = ⟨I²⟩(x, y) − [⟨I⟩(x, y)]², where ⟨·⟩ denotes the box-filter mean over a w × w neighborhood. Both terms are computed using scipy.ndimage.uniform_filter, yielding O(HW) complexity independent of window size. The raw variance map is post-smoothed with a separable Gaussian kernel (σ_smooth = 2.0 pixels) to suppress high-frequency noise while preserving the wound–cell transition.

Stage 2: Y-Band Detection. The wound is a horizontal band spanning nearly the full image width. Rather than searching the entire image, we first identify the vertical range [y_min, y_max] containing the wound. The row-averaged variance profile is computed and smoothed with a 1-D Gaussian kernel (σ_y = 10.0 pixels). The wound center is identified as the global minimum y_c. The profile is thresholded at the 35th percentile and the contiguous segment of sub-threshold rows containing y_c is extracted. The band is then expanded by a margin fraction m = 0.15.

Stage 3: Column-Wise Edge Extraction. Within the Y-band, upper and lower wound boundaries are extracted for each column by searching for the variance transition between cell (high σ²) and wound (low σ²) regions. The edge-detection threshold τ is derived adaptively as τ = 0.5 × median(σ²_cell), where σ²_cell is the set of variance values in the cell regions flanking the wound. For the upper edge, a downward search from y_min toward y_c finds the first row where variance drops below τ. For the lower edge, an upward search from y_max toward y_c performs the analogous operation. Both searches are fully vectorized using np.argmax on boolean arrays. Columns where no transition is found default to y_c.

Stage 4: RANSAC Outlier Rejection. Raw edges may contain outliers caused by imaging artifacts, debris, or partial cell detachment. An iterative fit-and-reject scheme inspired by RANSAC (Fischler & Bolles, 1981) is applied independently to each edge. A polynomial of degree 2 (parabolic) is fit to the edge, and points exceeding a maximum deviation of 25.0 pixels are replaced by the fitted values. Three iterations suffice for convergence in practice. Complexity is O(Wk) per edge, where k is the number of iterations.

Stage 5: Savitzky–Golay Smoothing. After outlier rejection, residual high-frequency jitter is smoothed with a Savitzky–Golay filter (Savitzky & Golay, 1964), which preserves edge shape without phase distortion. Remaining NaN values are filled by linear interpolation, and the filter is applied with window length w_sg = 51 and polynomial order p_sg = 3.

Stage 6: Mask Reconstruction. The binary mask is reconstructed from the smooth edges using vectorized broadcasting: M(x, y) = 1 if u(x) ≤ y ≤ ℓ(x), and 0 otherwise. This is a single-pass O(HW) operation.

Stage 7: Geometric Validation. Four quality-control criteria are evaluated on the resulting mask, as summarized in Table 1.

Table 1. Geometric validation criteria for single-frame wound masks.

| Criterion | Default Threshold | Rationale |
| --- | --- | --- |
| Width coverage (r_w) | r_w > 0.75 | Wound must span most of image width |
| Aspect ratio (ρ) | ρ > 1.5 | Wound is wider than tall |
| Y-position (ȳ_frac) | 0.15 < ȳ_frac < 0.85 | Wound is roughly centered |
| Thickness CV (CV_τ) | CV_τ < 0.60 | Thickness is consistent across columns |

A detection passes quality control if and only if all four criteria are satisfied simultaneously. Failures are logged with a diagnostic tag.

Fallback: Constrained Region Growing. When the primary pipeline fails validation, an optional fallback strategy is activated. The variance map is thresholded at the 25th percentile to create a low-variance mask. A thin horizontal seed (11 pixels tall) is placed at y_c and grown by intersection with the low-variance mask. Morphological cleanup follows: hole filling (area threshold 500 px), small-object removal (minimum size 1000 px), and closing with a disk structuring element (radius 5). Only the largest connected component is retained, and the result is re-validated with the same four geometric criteria.

## 2.3 Temporal Constraint: Monotonic Closure

The GeminiSegmenter wraps the single-frame WoundDetector with a stateful temporal integrator that enforces the biological axiom.

### Formal Definition

Let u_det(x, t) and ℓ_det(x, t) denote the raw detector output at time t. The constrained edges are defined recursively. At initialization (t = 0): u(x, 0) = u_det(x, 0) and ℓ(x, 0) = ℓ_det(x, 0). For t > 0:

u(x, t) = max(u(x, t−1), u_det(x, t))
ℓ(x, t) = min(ℓ(x, t−1), ℓ_det(x, t))

The upper edge can only move down (increase in image coordinates) and the lower edge can only move up (decrease). This guarantees that the wound width at every column is non-increasing over time, and consequently A(t) ≤ A(t−1) for all t > 0.

### Full Closure Handling

When the constrained edges meet or cross at a column (u(x, t) ≥ ℓ(x, t)), that column is declared fully closed with zero wound width. No artificial minimum gap is imposed, eliminating systematic area overestimation at late timepoints.

### State Management

The segmenter maintains three state vectors per trajectory, as summarized in Table 2.

Table 2. State vectors maintained by the GeminiSegmenter.

| State | Description | Updated at |
| --- | --- | --- |
| t0_upper, t0_lower, t0_mask | Reference frame (baseline) | t = 0 only |
| prev_upper, prev_lower, prev_mask | Previous frame | Every t |
| _t | Current time counter | Every call |

A call to reset() clears all state, preparing the segmenter for a new biological sample. This is essential: the monotonic constraint is defined within a trajectory, not across different samples.

## 2.4 Trajectory Processing Pipeline

### Per-Sample Workflow

For each biological sample (identified by a unique sample_name × experiment × exposure tuple): (1) Image files are discovered and sorted by acquisition time parsed from filenames. (2) Images are fed sequentially to the segmenter, which auto-increments t and applies the monotonic constraint for t > 0. (3) Per-frame results (image, mask, edges, area, QC flags) are serialized to experiment.pkl (full data) and summary.csv (lightweight table). (4) A two-panel trajectory plot is generated showing absolute wound area and normalized area (% of A(0)) versus time.

### Parallel Execution

Independent samples are processed in parallel using Python’s concurrent.futures.ProcessPoolExecutor. The number of worker processes is explicitly controlled (default 4, capped at the number of available CPU cores). Each worker instantiates its own GeminiSegmenter with independent state, ensuring no shared mutable data.

## 2.5 Quality Control

Quality control operates at three levels, from automated single-frame checks to optional human curation.

Level 1: Single-Frame Geometric Validation. Applied automatically to every frame. The four criteria described in Section 2.2 (Stage 7) flag frames with implausible geometry. Frames failing QC are either reprocessed via the fallback detector or flagged for downstream exclusion.

Level 2: Baseline (t = 0) Distributional Analysis. The baseline mask undergoes additional scrutiny. The Y-profile (row projection) of the mask, smoothed with a Gaussian kernel (σ = 15), should exhibit exactly one prominent peak with its center within ±15% of the mask centroid. The X-profile (column projection), normalized to [0, 1], should be roughly rectangular. Two criteria are evaluated: the absolute skewness must be less than 0.8, and the standard deviation of the central band (middle 50%) must be less than 0.15.

Level 3: Manual Curation (Optional). For high-stakes analyses, baseline images are exported to an annotation sheet for human review. A keep column flags samples for inclusion or exclusion, and downstream filtering respects these annotations.

## 2.6 Downstream Metrics

### Edge Displacement and Closure Speed

Given edges at t = 0 and a target time t_T, per-edge displacements are computed: Δu(x) = u(x, t_T) − u(x, 0) for the upper edge (positive = moved down), and Δℓ(x) = ℓ(x, 0) − ℓ(x, t_T) for the lower edge (positive = moved up). Speeds are computed as mean absolute displacements divided by elapsed time over the set of columns with valid (non-NaN) edges at both timepoints. The mean closure speed is the average of the upper and lower edge speeds.

### Wound Closing Determination

A trajectory is classified as “closing” if a minimum fraction of valid columns (f_min = 0.6) show inward edge motion at both boundaries.

### Analysis Modes

Two complementary analysis modes are provided. In cross-sectional mode, wound metrics at a single target timepoint are compared across experimental conditions (e.g., drug versus control at t = 20 h). In time-series mode, wound metrics at all available timepoints are tracked, enabling kinetic curve fitting and time-to-closure estimation.

## 2.7 Implementation Details

### Software Environment

Table 3. Software environment.

| Component | Version / Library |
| --- | --- |
| Language | Python 3.11 |
| Array operations | NumPy 1.24+ |
| Image I/O | OpenCV 4.x (cv2) |
| Filtering | SciPy (uniform_filter, gaussian_filter1d, savgol_filter) |
| Morphology | scikit-image |
| Data management | pandas, pickle |
| Visualization | matplotlib |
| Parallelism | concurrent.futures.ProcessPoolExecutor |

### Computational Complexity

All stages operate in O(HW) time or better, with processing dominated by the variance computation and mask reconstruction. For a typical 1024 × 1280 image, this corresponds to approximately 1.3 × 10⁶ operations per stage. Table 4 summarizes the per-stage complexity.

Table 4. Computational complexity per processing stage.

| Stage | Operation | Complexity |
| --- | --- | --- |
| 1. Local variance | Uniform filter (separable) | O(HW) |
| 2. Y-band | 1-D profile + argmin | O(H + W) |
| 3. Edge extraction | Boolean argmax, vectorized | O(HW) |
| 4. RANSAC | Polyfit × iterations | O(Wk) |
| 5. Savitzky–Golay | 1-D filter | O(W) |
| 6. Mask rebuild | Broadcasting | O(HW) |
| 7. Validation | Summary statistics | O(HW) |
| Monotonic constraint | Element-wise max/min | O(W) |

### Vectorization Strategy

All per-column operations that would naively require Python-level for-loops are implemented using NumPy broadcasting and fancy indexing. Edge extraction uses np.argmax on 2-D boolean arrays; mask reconstruction uses rank-broadcasting; and the monotonic constraint uses element-wise np.maximum and np.minimum on 1-D edge arrays.

# 3. Results

[Placeholder: experimental validation results to be inserted]

## 3.1 Processing Performance

[Table: wall-clock time per frame, per trajectory, throughput in trajectories/minute]

## 3.2 Segmentation Accuracy

[Table: comparison with manual annotations—Dice coefficient, Hausdorff distance, area correlation]

## 3.3 Reproducibility

[Table: intra-run and inter-run coefficient of variation for wound area measurements]

## 3.4 Effect of Monotonic Constraint

[Figure: wound area curves with and without temporal constraint, showing elimination of non-biological fluctuations]

## 3.5 QC Performance

[Table: QC pass rates by experimental condition, false positive/negative rates against manual curation]

# 4. Discussion

## 4.1 Advantages

The Gemini pipeline offers several methodological advances over existing tools:

No training data. The variance-based detector exploits a fundamental physical property of phase-contrast imaging (cells scatter light; the empty wound channel does not) rather than learned features. This eliminates the need for annotated training sets and avoids the domain-shift problem inherent in deep-learning segmenters.

Biologically grounded constraint. The monotonic closure axiom is not merely a regularizer; it encodes a physical law of the assay system. This produces strictly non-increasing area curves that are directly interpretable without post-hoc smoothing.

Computational efficiency. Full vectorization ensures that per-frame processing is dominated by array operations, not Python overhead. The O(HW) scaling enables real-time processing of standard microscopy images.

Modular design. The single-frame detector and temporal integrator are cleanly separated. The detector can be used independently for snapshot analyses, while the integrator can, in principle, wrap any detector that produces per-column edge arrays.

## 4.2 Limitations

Several known limitations warrant discussion:

Irreversible per-column constraint. If the detector produces a spurious inward edge at time t (due to debris, bubble, or focus artifact), the monotonic constraint permanently propagates that error to all subsequent frames for the affected columns. There is no relaxation or error-correction mechanism. Spatially smoothing the edges before applying the constraint or adopting a confidence-weighted approach could mitigate this.

Fixed threshold heuristic. The edge-detection threshold (τ = 0.5 × median(σ²_cell)) assumes unimodal cell variance and a clean cell–wound boundary. Images with heterogeneous cell populations or illumination gradients may require adaptive thresholding.

Integer-pixel edges. Edges are extracted at integer pixel resolution. For thin wounds approaching closure, the 1-pixel quantization error represents a significant fraction of the remaining width. Sub-pixel edge localization would improve precision.

No uncertainty quantification. The pipeline produces point estimates of wound area without confidence intervals. Bootstrapping area estimates under small edge perturbations would provide uncertainty bounds.

No physical calibration. All measurements are in pixel units. Incorporating a pixel-to-micrometer conversion factor would yield physically meaningful areas and speeds.

## 4.3 Future Directions

Planned improvements include: (1) adaptive per-column thresholding using local statistics or Otsu’s method; (2) sub-pixel edge extraction via parabolic fitting; (3) a Kalman filter combining the detector output (observation) with the monotonic prediction (prior), weighted by per-frame confidence; (4) bootstrap-based uncertainty quantification; and (5) integration of pixel-size metadata for calibrated measurements.

# 5. Conclusion

We have presented Gemini, a fully automated pipeline for scratch-wound healing quantification that combines variance-based single-frame detection with a biologically motivated monotonic closure constraint. The pipeline requires no training data, processes standard phase-contrast microscopy images in linear time, and produces smooth, strictly non-increasing wound area trajectories. A multi-tier quality-control system ensures robust performance, while parallel trajectory processing enables scalable analysis of high-content experiments. The mathematical formulation is presented in full, all parameters are documented, and the implementation is released as open-source Python code. We anticipate that Gemini will reduce the time and subjectivity currently associated with scratch-wound quantification, enabling researchers to focus on biological interpretation rather than manual measurement.

# References

1. Fischler, M. A., & Bolles, R. C. (1981). Random sample consensus: a paradigm for model fitting with applications to image analysis and automated cartography. Communications of the ACM, 24(6), 381–395.

2. Gebäck, T., Schulz, M. M. P., Koumoutsakos, P., & Detmar, M. (2009). TScratch: a novel and simple software tool for automated analysis of monolayer wound healing assays. BioTechniques, 46(4), 265–274.

3. Jonkman, J. E., Cathcart, J. A., Xu, F., Bartolini, M. E., Amon, J. E., Stevens, K. M., & Bhargava, P. (2014). An introduction to the wound healing assay using live-cell microscopy. Cell Adhesion & Migration, 8(5), 440–451.

4. Liang, C.-C., Park, A. Y., & Guan, J.-L. (2007). In vitro scratch assay: a convenient and inexpensive method for analysis of cell migration in vitro. Nature Protocols, 2(2), 329–333.

5. Savitzky, A., & Golay, M. J. E. (1964). Smoothing and differentiation of data by simplified least squares procedures. Analytical Chemistry, 36(8), 1627–1639.

6. Suarez-Arnedo, A., Torres Figueroa, F., Clavijo, C., Arbelaez, P., Cruz, J. C., & Munoz-Camargo, C. (2020). An image J plugin for the high throughput image analysis of in vitro scratch wound healing assays. PLOS ONE, 15(7), e0232565.

7. Vu, Q. D., et al. (2023). Deep learning for wound healing assay segmentation. [Journal placeholder].

# Supplementary Materials

## Table S1: Full Parameter Table

| Parameter | Default | Type | Stage | Description |
| --- | --- | --- | --- | --- |
| variance_window | 15 | int | 1 | Side length of local variance kernel (pixels) |
| variance_sigma | 2.0 | float | 1 | Gaussian post-smoothing sigma for variance map |
| y_profile_sigma | 10.0 | float | 2 | Gaussian sigma for Y-profile smoothing |
| y_band_percentile | 35.0 | float | 2 | Percentile threshold for Y-band detection |
| y_margin | 0.15 | float | 2 | Fractional expansion of detected Y-band |
| edge_threshold_percentile | 40.0 | float | 3 | Fallback percentile when cell regions are empty |
| ransac_poly_degree | 2 | int | 4 | Polynomial degree for RANSAC fit |
| ransac_max_deviation | 25.0 | float | 4 | Max pixel deviation before outlier classification |
| ransac_iterations | 3 | int | 4 | Number of fit-reject-refit cycles |
| smoothing_window | 51 | int | 5 | Savitzky–Golay filter window (must be odd) |
| smoothing_polyorder | 3 | int | 5 | Savitzky–Golay polynomial order |
| min_width_coverage | 0.75 | float | 7 | Minimum fraction of columns with wound pixels |
| min_aspect_ratio | 1.5 | float | 7 | Minimum bounding-box width / height |
| max_thickness_cv | 0.60 | float | 7 | Maximum coefficient of variation of thickness |
| y_center_range | (0.15, 0.85) | tuple | 7 | Acceptable range for wound Y-center (frac of H) |
| fallback_enabled | True | bool | 7+ | Enable region-growing fallback |

## Table S2: Quality Control Criteria

| Level | Criterion | Threshold | Applied to |
| --- | --- | --- | --- |
| 1 (per-frame) | Width coverage | > 0.75 | Every frame |
| 1 (per-frame) | Aspect ratio | > 1.5 | Every frame |
| 1 (per-frame) | Y-center position | ∈ (0.15, 0.85) | Every frame |
| 1 (per-frame) | Thickness CV | < 0.60 | Every frame |
| 2 (baseline) | Y-profile peak count | = 1 | t = 0 only |
| 2 (baseline) | Y-profile peak offset | < 0.15 × H | t = 0 only |
| 2 (baseline) | X-profile skewness | < 0.80 | t = 0 only |
| 2 (baseline) | X-profile central flatness | < 0.15 | t = 0 only |
| 3 (manual) | Human annotation | keep = True | Optional |

## Table S3: Known Limitations

| ID | Category | Severity | Description |
| --- | --- | --- | --- |
| W3 | Robustness | Medium | Fixed threshold heuristic not adaptive to heterogeneous images |
| W4 | Robustness | Medium | Single global minimum for Y-band; fails with multiple low-variance bands |
| W8 | Correctness | High | Per-column monotonic constraint is irreversible; one artifact permanently biases subsequent frames |
| W9 | Precision | Medium | Integer-pixel edge resolution; quantization error significant for thin wounds |
| W11 | Precision | Low | No temporal smoothing of detection results within monotonic bounds |
| W13 | Statistical | High | No uncertainty quantification; point estimates without confidence intervals |
| W14 | Analytical | Medium | No pixel-to-micrometer calibration; all measurements in pixel units |